- The Fourier transform of an analogue signal x(t) is given by:

- The Discrete Fourier Transform (DFT) of a discrete-time signal x(nT) is given by:

Where:

x[n] 	= input

X[k] 	= frequency bins

X(0) 	= x[0]WN0 + x[1]WN0*1 +…+ x[N-1]WN0*(N-1)

X(1) 	= x[0]WN0 + x[1]WN1*1 +…+ x[N-1]WN1*(N-1)

:

X(k) 	= x[0]WN0 + x[1]WNk*1 +…+ x[N-1]WNk*(N-1)

:

X(N-1)	= x[0]WN0 + x[1]WN (N-1)*1 +…+ x[N-1]WN (N-1)(N-1)

W	= twiddle factors

Note: For N samples of x we have N frequencies representing the signal.

Performance of the DFT Algorithm

- The DFT requires N2 (NxN) complex multiplications:

  - Each X(k) requires N complex multiplications.

  - Therefore to evaluate all the values of the DFT ( X(0) to X(N-1) ) N2 multiplications are required.

- The DFT also requires (N-1)*N complex additions:

  - Each X(k) requires N-1 additions.

  - Therefore to evaluate all the values of the DFT (N-1)*N additions are required.

- Can the number of computations required be reduced?

DFT  FFT

- A large amount of work has been devoted to reducing the computation time of a DFT.

- This has led to efficient algorithms which are known as the Fast Fourier Transform (FFT) algorithms.

(1)

x[n] = x[0], x[1], …, x[N-1]

- Lets divide the sequence x[n] into even and odd sequences:

  - x[2n] 	= x[0], x[2], …, x[N-2]

  - x[2n+1] 	= x[1], x[3], …, x[N-1]

- Equation 1 can be rewritten as:

(2)

- Since:

- Then:

- The result is that an N-point DFT can be divided into two N/2 point DFT’s:

N-point DFT

- Where Y(k) and Z(k) are the two N/2 point DFTs operating on even and odd samples respectively:

Two N/2-point DFTs

4 point DIT-FFT

Example 1: Consider a sequence x[n]={ 1, 1, -1, -1, -1, 1, 1, -1}

Determine DFT X[k] of x[n] using the decimation-in-time FFT algorithm.

fn= x2n=  x0, x2, x4, x6    = {1, -1, -1, 1}

g[n]= x[2n+]= { x[1], x[3], x[5], x[7]}= {1, -1, 1, -1}

X= Wx

F0F(1)F(2)F(3)=w40w40w40w40w40w41w42w43w40w42w44w46w40w43w46w49   f0f(1)f(2)f(3)

=11111-j-1j1-11-11j-1-j  1-1-11

=02+j202-j2

G0G(1)G(2)G(3)=w40w40w40w40w40w41w42w43w40w42w44w46w40w43w46w49   g0g(1)g(2)g(3)

=11111-j-1j1-11-11j-1-j  1-11-1

=0040

X0=F0+W80G0=0                                       X4=F0-W80G0=0

X1=F1+W81G1=2+j2                              X5=F1-W81G1=2+j2

X2=F2+W82G2=-j4                                 X6=F2-W82G2=j4

X3=F3+W83G3=2-j2                             X7=F3-W83G3=2-j2

Example 1: Consider a sequence x[n]={ 1, 1, -1, -1, -1, 1, 1, -1}

Determine DFT X[k] of x[n] using the decimation-in-frequency FFT algorithm.

pn= xn+xn+N2

=  1-1, 1+1, -1+1, (-1-1)    = {0, 2, 0, 2}

qn= xn-xn+N2W8n

=  1+1W80, 1-1W81, -1-1W82, (-1+1)W83

= {2, 0, j2, 0}

X=Wx

P0P(1)P(2)P(3)=w40w40w40w40w40w41w42w43w40w42w44w46w40w43w46w49   p0p(1)p(2)p(3)

=11111-j-1j1-11-11j-1-j  020-2

=0-j40j4

Q0Q(1)Q(2)Q(3)=w40w40w40w40w40w41w42w43w40w42w44w46w40w43w46w49   q0q(1)q(2)q(3)

=11111-j-1j1-11-11j-1-j  20j20

=2+j22-j22+j22-j2

X0=P0=0                                                         X4=P2=0

X1=Q0=2+j2                                                X5=Q2=2+j2

X2=P1+W82G2=-j4                                 X6=P3=j4

X3=Q1+W83G3=2-j2                             X7=Q3=2-j2